Chileaf BLE Protocol

(V0.6)

1.	Definition of Bluetooth UUID	- 1 -

1.1.	Bluetooth broadcast name	- 1 -

1.2.	Bluetooth broadcast Manufacturer data	- 1 -

1.3.	Service and characteristics	- 1 -

2.	Definition of data format	- 3 -

2.1.	Data format	- 3 -

2.2.	Frame field description	- 3 -

2.3.	Checksum calculation method	- 3 -

3.	Definition of command	- 4 -

3.1.	System commands	- 4 -

3.1.1.	Get function list and protocol version-0x01(reserve)	- 4 -

3.1.2.	Get user info and status information-0x03	- 4 -

3.1.3.	Configure user information-0x04	- 4 -

3.1.4.	Set UTC time (with time zone)-0x08	- 5 -

3.2.	Heart rate and step command	- 5 -

3.2.1.	Real-time sports data notification-0x15	- 5 -

3.2.2.	Get 7days exercise data-0x16	- 6 -

3.2.3.	Heart rate history data list request-0x21	- 6 -

3.2.4.	Heart rate history data request-0x22	- 7 -

3.2.5.	Acceleration 3D raw data-0x0C(Optional)	- 7 -

3.2.6.	SPO2 Mode-0x37	- 8 -

3.2.7.	Temperature	- 8 -

3.3.	Rope skipping	- 9 -

3.3.1.	Rope status-0x40	- 9 -

3.3.2.	Realtime rope notify-0x41	- 10 -

3.3.3.	Set mode-0x42	- 10 -

3.3.4.	Clear-0x45	- 11 -

| version | Date | Editor | reviewer | Description |
| --- | --- | --- | --- | --- |
| V0.1 | 2019.10.09 | jzhou |  | First release |
| V0.2 | 2019.10.14 | jzhou |  | Bluetooth broadcast data. |
| V0.3 | 2020.3.20 | Rickon |  |  |
| V0.4 | 2021.9.10 | Rickon |  |  |

- Definition of Bluetooth UUID

  - Bluetooth broadcast name

[Model name]-xxxxxxx, where xxxxxxx is a 7-bit id code

  - Bluetooth broadcast Manufacturer data

| Manufacturer data | Data type description |
| --- | --- |
| Manufacturer identification ID | 0xff04 |
| Byte 0 | Reserve |
| Byte 1 | Battery level (0~100%) |
| Byte 2 | Reserve |
| Byte 3 | Heart rate data |

  - Service and characteristics

- Custom service UUID:

aae28f00-71b5-42a1-8c3c-f9cf6ac969d0

| type | Attribute UUID | description | attribute |
| --- | --- | --- | --- |
| service | 8f00 |  |  |
| TX characteristics | 8f01 | Read data from device | NOTIFY |
| RX characteristics | 8f02 | Write data to device | WRITE |

- Heart rate service

| type | Attribute UUID | description | attribute |
| --- | --- | --- | --- |
| Heart rate service | 180d |  |  |
| Heart rate data | 2a37 | Device notification | NOTIFY |
| Wear position | 2a38 | Read data from device | READ |

- The Device Information service

| type | Attribute UUID | description | attribute |
| --- | --- | --- | --- |
| The DEVICE service | 180a |  |  |
| Name of manufacturer | 2a29 | Read data from device | READ |
| The Model name | 2a24 | Read data from device | READ |
| Serial number | 2a25 | Read data from device | READ |
| Hardware version | 2a27 | Read data from the device | READ |
| The Firmware version | 2a26 | Read data from the device | READ |
| Software version | 2a28 | Read data from the device | READ |

- Battery service

| type | Attribute UUID | description | attribute |
| --- | --- | --- | --- |
| The DEVICE service | 180f |  |  |
| battery | 2a19 | Read and notify data | READ, NOTIFY |

Note: the APP can read the battery level directly, also device can notify it.

- Definition of data format

  - Data format

| Head | length | Command | Data | Checksum |
| --- | --- | --- | --- | --- |
| 0xFF | N + 4 | 0x** | -- | checksum |
| 1 byte | 1 byte | 1 byte | N bytes | 1 byte |

  - Frame field description

| Item | byte | type | description | The instance |
| --- | --- | --- | --- | --- |
| Head | 1 | HEX | Pack start byte | 0xFF |
| Length | 1 | HEX | From head to checksum | 0x04 + N |
| Command | 1 | HEX | Function of the command | 0 x01 |
| Data | N | HEX | The data content | -- |
| Checksum | 1 | HEX | The sum of whole pack except checksum | checksum |

  - Checksum calculation method

- First calculate the sum of head/length/command/data.

- Then subtract the sum from 0;

- Finally XOR 0x3a, and take the low 8-bit value as the value of checksum.

The following is the calculation template for reference

Public byte calcChecksum(byte[] dat) {

Int I = 0;

Byte res = 0;

Int len = dat.length-1;

For (I = 0;   I <len.   I++) {

Res + = dat [I];

}

Int temp = (int) res;

Temp & = 0xFF;

Temp = (0 - temp);

Temp & = 0xFF;

Temp ^ = 0x3a;

Res = (byte) (temp & 0xff);

Return res;

}

- Definition of command

  - System commands

    - Get function list and protocol version-0x01(reserve)

| Data bits | Command from APP | Reply from device | Functional description |
| --- | --- | --- | --- |
| Command | 0x01 | 0x01 | Get the feature list and protocol version |
|  |  |  | reserve |

    - Get user info and status information-0x03

| Data bits | APP request | Device reply | Functional description |
| --- | --- | --- | --- |
| Command | 0x03 | 0 x03 | Gets user and status information |
| Data segment | None | 0 x * * | ECG is open: 0 is not open, 1 is open |
|  |  | 0 x * * | Device charging information: 0: not charged; 1: in charge; 2:The battery is fully charged while charging |
|  |  | 0 x * * | The percentage of battery power is 0~100% |
|  |  | 0 x * * | User age |
|  |  | 0 x * * | User gender: 0-female, 1-male |
|  |  | 0 x * * | User weight: unit: kg |
|  |  | 0 x * * | User height: unit: cm |
|  |  | 0 x * * | User phone number: high byte |
|  |  | 0 x * * | User phone number: |
|  |  | 0 x * * | User phone number: |
|  |  | 0 x * * | User phone number: |
|  |  | 0 x * * | User phone number: low byte |

    - Configure user information-0x04

| Data bits | APP send | Device reply | Functional description |
| --- | --- | --- | --- |
| Command | 0x04 | 0x04 | Configure user information |
|  | 0 x * * | NA | User age |
|  | 0 x * * |  | User gender: 0-female, 1-male |
|  | 0 x * * |  | User weight: unit: kg |
|  | 0 x * * |  | User height: unit cm |
|  | 0 x * * |  | User phone number: high byte |
|  | 0 x * * |  | User phone number: |
|  | 0 x * * |  | User phone number: |
|  | 0 x * * |  | User phone number: |
|  | 0 x * * |  | User phone number: low byte |

    - Set UTC time (with time zone)-0x08

| Data bits | APP send | Device reply | Functional description |
| --- | --- | --- | --- |
| Command | 0x08 | 0x08 | Configure user information |
|  | 0 x * * |  | UTC: high byte |
|  | 0 x * * |  | UTC: |
|  | 0 x * * |  | UTC: |
|  | 0 x * * |  | UTC: low byte |

  - Heart rate and step command

    - Real-time sports data notification-0x15

| Data bits | APP reply | device notify | Functional description |
| --- | --- | --- | --- |
| Command |  | 0 x15 |  |
|  |  | 0 x * * | Step high byte |
|  |  | 0 x * * | Step middle byte |
|  |  | 0 x * * | Step low byte |
|  |  | 0 x * * | Distance high byte –unit: cm |
|  |  | 0 x * * | Distance middle byte |
|  |  | 0 x * * | Distance low byte |
|  |  | 0 x * * | Calorie high – unit: 0.1 Kcal |
|  |  | 0 x * * | Calorie middle |
|  |  | 0 x * * | Calorie low |

    - Get 7days exercise data-0x16

| Data bits | APP request device | Device to the APP | Functional description |
| --- | --- | --- | --- |
| Command | 0 x16 | 0x16 | Seven days of historical data |
|  | NA | 0 x * * | UTC: high |
|  |  | 0 x * * | UTC: |
|  |  | 0 x * * | UTC: |
|  |  | 0 x * * | UTC: low |
|  |  | 0 x * * | Step of one day(n) high |
|  |  | 0 x * * | Step of one day: |
|  |  | 0 x * * | Step of one day: low |
|  |  | 0 x * * | Calories of one day (n) : high |
|  |  | 0 x * * | Calories of one day: |
|  |  | 0 x * * | Calories of one day: low |
|  |  | … | … |

    - Heart rate history data list request-0x21

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
| Command | 0 x21 | 0 x21 | Request a list of heart rate history data |
|  |  | 0 x * * | UTC 0: high |
|  |  | 0 x * * | UTC 0: |
|  |  | 0 x * * | UTC 0: |
|  |  | 0 x * * | UTC 0: low |
|  |  | ... |  |
|  |  | 0 x * * | UTC n: |
|  |  | 0 x * * | UTC n: |
|  |  | 0 x * * | UTC n: |
|  |  | 0 x * * | UTC n: |

Note: UTC time means that there is a pack of heart rate data at the beginning of this time.

If there is no historical data, a UTC time 0xffffffff reply.

    - Heart rate history data request-0x22

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
| Command | 0 x22 | 0 x22/0 x23 | Request heart rate history |
|  | - Request single data - Request all data(reserve) - Request all data after utc(reserve) | 0 x * * | UTC or package number: high |
|  | UTC n: high | 0 x * * | UTC or package number: |
|  | UTC n: | 0 x * * | UTC or package number: |
|  | UTC n: | 0 x * * | UTC or package number: |
|  | UTC n: low | 0 x * * | Heart rate data 0 |
|  |  | 0 x * * | Heart rate data 1 |
|  |  | 0 x * * | Heart rate data n |

Note:

1:APP needs to request the heart rate list(0x21) first and get all the UTC time of records, and then use the UTC time to request data;

2: If package end or no history, device will send 0x23 command to indicate it;

3: The first package of each history data will include UTC time, from the second package it send package number instead of UTC.

|  |  | 0 x * * | UTC or package number: |
| --- | --- | --- | --- |
|  |  | 0 x * * | UTC or package number: low |
|  |  | 0 x * * | ACT-0: 5 minute action index |
|  |  | 0 x * * | ACT-1:5-minute action index |
|  |  | 0 x * * | ACT-2: 5 minute action index |
|  |  | 0 x * * | ACT-n: 5 minute action index |

    - Acceleration 3D raw data-0x0C(Optional)

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
|  |  | 0x0C | 3D raw data |
|  |  | 0x** | Xl1: low byte |
|  |  | 0x** | Xh1: hight byte |
|  |  | 0x** | Yl1 |
|  |  | 0x** | Yh1 |
|  |  | 0x** | Zl1 |
|  |  | 0x** | Zh1 |
|  |  | 0x** | Xl2 |
|  |  | 0x** | Xh2 |
|  |  | 0x** | ……….. |
|  |  | 0x** | zln |
|  |  | 0x** | zhn |

note：

Send every 250ms

    - SPO2 Mode-0x37

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
| Command | 0x37 | 0x37 |  |
|  | 0x** | 0x** | 0: exit spo2 mode 1: enter spo2 mode 2:inquire |
|  |  | 0x** | SPO2 value |
|  |  | 0x** | 0：wrist posture wrong 1：wrist posture correct(face up) |
|  |  | 0x** | 0: no signal <8: signal week >15:signal good |
|  |  | 0x** | 0: not wear 1:wear |

    - Temperature

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
| Command | 0x38 | 0x38 | Real time temperature （unit：*10℃） |
|  |  | 0x** | Ambient temperature (MSB) |
|  |  | 0x** |  |
|  |  | 0x** | Wrist temperature (MSB) |
|  |  | 0x** |  |
|  |  | 0x** | body temperature (MSB) |
|  |  | 0x** |  |

  - Rope skipping

    - Rope status-0x40

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
| command | 0x40 | 0x40 |  |
| 1 |  | UTC | MSB |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  | MODE | 0:FREE ;1:COUNTER ;2:TIMER |
| 6 |  | COUNTER: target | < 9999(MSB) |
| 7 |  |  |  |
| 8 |  | COUNTER:jumps | MSB |
| 9 |  |  |  |
| 10 |  | COUNTER:TIME | MINUTE |
| 11 |  |  | SECOND |
| 12 |  | COUNTER:Calorie | MSB |
| 13 |  |  |  |
| 14 |  | TIMER:Target time | Minute(<99) |
| 15 |  |  | second |
| 16 |  | TIMER:Current time | Minute |
| 17 |  |  | second |
| 18 |  | TIMER:Jumps |  |
| 19 |  |  |  |
| 20 |  | TIMER:Calorie | MSB |
| 21 |  |  |  |
| 22 |  | FREE:Jumps |  |
| 23 |  |  |  |
| 24 |  | FREE:TIME | MINUTE |
| 25 |  |  | SEDOND |
| 26 |  | FREE:Calorie | MSB |
| 27 |  |  |  |
| 28 |  | Jumps of day |  |
| 29 |  |  |  |
| 30 |  | Time of day |  |
| 31 |  |  |  |
| 32 |  | Calorie of day |  |
| 33 |  |  |  |

    - Realtime rope notify-0x41

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
| command |  | 0x41 |  |
| 1 |  | Current MODE |  |
| 2 |  | Current jumps |  |
| 3 |  |  |  |
| 6 |  | Time |  |
| 7 |  |  | ms |
| 8 |  | Calorie |  |
| 9 |  |  |  |
| 10 |  | elapsed time | Minute |
| 11 |  |  | Second |
| 12 |  | Current count down time | Minute |
| 13 |  |  | Second |
| 14 |  | If restart | 1：restart。0：not restart |
| 15 |  | Trip | msb |
| 16 |  |  |  |

    - Set mode-0x42

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
| command | 0x42 | 0x42 | 0x42 |
| 1 | mode | 0,1,2 | FREE/COUNTER/TIMER |
| 2 | Target counter |  |  |
| 3 |  |  |  |
| 4 | Timer | Minute |  |
| 5 |  | second |  |

    - Clear-0x45

| Data bits | The APP requests | Device reply APP | Functional description |
| --- | --- | --- | --- |
| command | 0x45 | 0x45 | clear |
| 1 |  |  |  |